# AI in the Wild - Group Project Report

Gesture and Vision Control for DJI Tello Drone

| Team Members | KyleDonovan69 & Josh-Alsybury (50/50 work split) |
| --- | --- |
| Date | 20th April 2026 |

# The Process

## What We Set Out to Achieve

Our goal was to build a drone system that lets you control the DJI Tello drone through a few different AI methods like:

- hand gestures,

- automatically following a person,

- and hunting down a specific object.

All of it accessible from one console, no need to touch a keyboard once it's running.

## What We Did

### Gesture Recognition

We built our own data collection tool using ‘MediaPipe Hands’ to record 21 hand landmarks per frame, which gives you 42 x/y coordinates to work with.

We recorded 100 samples each for 8 different gestures, so around 800 total, and trained a MLP neural network classifier on that.

The gestures we used were:

- Open Palm (hover)

- Fist (land)

- Point Up and Down(elevation), Point Left and Right (rotation)

- Peace Sign (cycle mode)

- Thumbs Up (confirm mode)

We also added a 15-frame buffer so the drone doesn't react to anything in-between.

### Person Following

This uses the MediaPipe Pose to track a person's shoulders and hips.

The drone adjusts its yaw to keep them centred, then moves up or down based on where they are in frame, and goes forward or back based on how much of the frame they take up (so it keeps a consistent distance and doesn’t get too close/far).

We added dead zones and capped commands at 7Hz to stop it from constantly twitching.

### Hunt Mode

Hunt mode looks for Monster energy cans using HSV colour detection.

It works in stages: first it does a loose detection to find anything roughly the right colour, then approaches, then does a strict check where it has to hover for a second and confirm the target is at least 45% green.

If it loses the target it runs a square patrol pattern, doing a full 360 scan at each corner.

We also filter by aspect ratio so it doesn't get fooled by things like green bags.

### Recall / Return Function

Every RC command the drone sends gets saved as a breadcrumb (left/right, forward/back, up/down, yaw, and duration).

When you hit RECALL HOME it plays those back in reverse at 60% speed, skipping yaw and up/down to avoid compounding any drift.

In practice it ended up working more as an emergency stop than a real return-to-home, but it still gets the drone down safely which is the important part.

### Drone Simulator

Since we didn't have access to the actual drone outside of class and didn’t have 300 euro to buy one I built a simulator as a basic replacement.

It uses your laptop webcam as the video source and then just prints all the RC commands to the terminal instead of actually sending them.

This meant we could develop and test all the AI stuff without needing the real hardware, which saved a huge amount of time.

### GUI and Base Code

We used Tkinter for the GUI, with a live video panel, manual control buttons, and toggles for each mode.

Background threading keeps the video feed separate from the command loop so the interface stays responsive.

The whole codebase was set up from the start to keep things separate, the gesture training pipeline, drone app, simulator, and person follower all live as their own modules so you can work on one without breaking another.

### Safety

We have a few small safety features in the project:

- The drone auto-lands when battery drops to 15%.

- The commands are capped at 7Hz to stop the queue from overflowing.

- If the drone loses its target it just hovers and stops moving.

There's also the RECALL HOME kill switch and you can just physically flip the drone upside down as a last resort but mind your fingers, it actually hurts quite a bit if it gets you.

# The Result

## What Was Achieved

We ended up with a fully working multi-mode drone system that actually does what we set out to make it do.

All four modes ran pretty reliably in lab conditions and the whole thing is accessible from one GUI.

### Gesture Recognition

All 8 gestures get picked up accurately with our custom dataset.

The 15-frame buffer does a good job of stopping the false triggers while still feeling responsive enough to use.

### Person Following

The drone tracks a walking person pretty smoothly and also follows them up and down if they crouch.

It keeps a consistent distance and stays centred on them.

The dead zones and 7Hz cap sorted out the drifting issues we had early on.

### Hunt Mode

It reliably finds and approaches Monster cans.

The two-stage confirmation stopped all the false positives we were getting with colours, and the square patrol covers the room well when there's no target in sight.

### Recall Function

Honestly it didn't really return the drone to where it started in a properly useful way.

It worked more as a kill switch that gets the drone down safely, which is fine, it just wasn't what we originally planned.

### Safety Systems

Battery auto-land at 15% worked consistently.

The command throttling stopped any queue overflow issues.

The kill switch does exactly what you'd want it to do.

## What Didn't Work

- The HSV ranges we started with were completely wrong for our lab lighting, we had to tune everything a few times manually.

- Without dead zones the person-following just drifted and floated around constantly.

- Including yaw in the recall reversal made the drone spin and drift in weird ways.

- Approaching a target immediately without confirming it first meant it chased green bags and jackets.

- Using full-body bounding boxes for person-following meant it would lock onto random people walking through the background, it would also get very close without the frame size checks.

- The original C++ H.264 decoder broke on Windows with Python 3.10, we had to swap it out for PyAV.

# 3. The Learnings

## Key Takeaways

### Real hardware doesn't behave like you expect.

WiFi latency changes, battery level affects how the motors respond and behaviours work, and the lighting in the room will mess with your colour detection.

You need to build in fallbacks for everything because something will go wrong even in best case scenarios.

Dead zones and throttling aren't optional, the drifting proved it.

We thought they were nice to have at first but after a few tests we realized they're not.

Without them the drone reacted to single-pixel changes and just drifted the whole time.

Once we added them and capped at 7Hz it went from unusable to actually smooth.

### Waiting for confirmation is worth it.

The 1-second hover and strict check in hunt mode felt like overkill at first but it completely fixed the false positives.

Going slower and being right is way better than being fast and wrong when there are spinning rotors involved (especially when its making a break for you).

### Just collect your own data.

We spent ages trying to get existing classifiers to work before realising we could just collect 800 samples ourselves in about 30 minutes.

The collection tool took maybe 2 hours to build and paid off straight away.

We went back and added more data on top of that too which helped a lot.

Plus in the end our data worked out better since it was trained in both similar lighting and with our actual bodies.

### Build a simulator early.

Every bit of lab time we had went towards testing things that actually need the hardware.

All the basic logic was already sorted before we even got near the drone.

This was especially useful for person-following and hunt mode since the real world conditions were pretty different from what the simulator could replicate (the simulator also has the issue of not being able to move the actual laptop).

Proportional control is much better than binary control.

Early on the drone just went full speed or stopped, which caused constant overshooting and was a bit dangerous.

Switching to proportional RC commands based on how far off-centre things are made the movement way smoother.

The breadcrumb idea was good in theory.

Skipping yaw and up/down in the recall to avoid compounding errors made sense on paper, but in practice it just didn't bring the drone back anywhere meaningful.

Still useful as an emergency stop though, so we kept it in for that reason.

# Division of Labour

The work was split 50/50 between both of us.

| Team Member | Contributions |
| --- | --- |
| KyleDonovan69 | Person follow mode, drone simulator for offline testing, background threading, base code structure, GUI implementation |
| Josh-Alsybury | Hunt mode (patrol, phase system, detection tuning), recall/return function, gesture data collection and training, mode system |

Both members contributed to building, testing, debugging and safety systems.